# Anomaly Detection in IoT Telemetry

Azure IoT Workshop

By Jevgenijs Pankovs

# Introduction

The goal of this workshop is to show you in minute details how to implement a typical IoT solution, fulfilling the following requirements:

- An IoT device, sending telemetry data, shall be connected to an Azure IoT Hub

- The telemetry data shall be processed with Azure Stream Analytics service to detect anomalies – spikes and dips or change points

- Visualize telemetry data and anomalies with Azure Time Series Insights

- Visualize telemetry and anomalies with Power BI

- Egress Stream Analytics output to an Azure Function to process anomalies.

The following diagram reflects different Azure components that will be used to architect our IoT solution:

Figure 1. Diagram of the architecture

First, we will deploy all the components to the Cloud manually. Then, we will be using a deployment script, creating the same components, to avoid repeating manual operations, to simplify and speed up our work.

Though, we could use a real IoT device (like a Raspberry Pi), we will create a device simulator using Visual Studio Code, Azure IoT SDK and Python or C# languages.

The amount of work is large, that’s why we will split it to a few parts. Here is our plan:

Part I:

- Create a resource group in Azure portal

- Create an IoT hub

- Create a Device Provisioning service, and link it to the IoT hub

- Create an IoT device simulator, connect it to the IoT hub, and send telemetry data

- Use Azure IoT Explorer to view the telemetry the simulated device is sending

Part II:

- Create a Stream Analytics job, and configure its inputs, outputs, and queries

- Create a Time Series Insights environment, and configure its event sources

- Visualize telemetry data in Time Series Insights

- Visualize data from Time Series Insights in Power BI

Part III:

- Create a script, deploying Azure resources

Part IV:

- Create an Azure Function and trigger it in response to Stream Analytics events.

Enough with the introduction, let’s start.

# Hands-on Lab. Part I

## 1. Create a resource group

Let us first create an Azure Resource Group, which will be a logical container for all our components. All the resources in a resource group will have the same life cycle. That means when a resource group is deleted, all resources in the group are deleted as well. The group needs a location to specify where the metadata will be stored, which is necessary for some compliance policies.

- Open Azure portal on the browser by navigating to http://portal.azure.com

- From the left top side of the portal home page click + Create a resource and select Resource group from the features options

- Select your Subscription, enter anomaly-detection-rg as a Resource group name, select North Europe Region, and then select Review + Create

- On the Summary page, select Create.

Once the deployment is complete, select Go to resource to open new resource group. Click + Create to create a new resource:

## 2. Create an IoT hub

Now, we will create an IoT hub, acting as a central message hub for communication between our IoT applications and its attached devices.

- Select IoT Hub from the list of resources

- Enter anomaly-detection-hub as a name of the IoT hub, and then select North Europe Region

- Press Next: Networking, and then select Next: Management to continue creating your hub.

- Leave S1: Standard tier as Pricing and scale tier

- Select Review + Create and click Create.

Once the IoT hub is created, select Go to resource to open the hub.

IoT Hub exposes a built-in endpoint to read the device-to-cloud messages received by your hub. IoT Hub enables you to create consumer groups on the endpoint, enabling multiple consuming applications to read the stream independently.

We are going to create a new consumer group for Azure Stream Analytics service, reading telemetry data from our IoT hub.

- Go to Built-in endpoints tab

- Enter asaconsumergroup as a new Consumer Group.

## 3. Create a Device Provisioning Service

Azure IoT Service offers many ways to connect an IoT device to your IoT hub. For example, you can register your devices in the IoT hub manually and connect them using symmetric keys or X.509 certificates. But if you need to provision thousands (or even millions) of devices, that’s not the case.

We need to realize a scenario when a connected IoT device is automatically registered in our IoT hub. That is why we will use Device Provisioning Service (DPS), supporting IoT Plug and Play architecture. Once a device starts communicating using Plug and Play protocol, it is automatically registered (if it was not) in the IoT hub. That is what we need. But first, let’s create a DPS.

- In the Azure portal, select + Create a resource, and select IoT Hub Device Provisioning Service

- Select your Subscription, select anomaly-detection-rg as a Resource group, and North Europe as a Region

- Enter anomaly-detection-dps as a Name and then select Review + Create

- On the Review + Create page, select Create.

- Once the DPS instance is created, open it, and go to Linked IoT hubs tab

- Click + Add button to link our previously created IoT hub

- On the Add link to IoT hub panel, select your Subscription

- Select anomaly-detection-hub as a name of the IoT hub

- Select iothubowner as Access Policy, and then click Save.

We’ve just linked our IoT hub to the DPS. If you do not see your hub in the list of linked hubs, click Refresh.

Now, we need to create a DPS Enrollment Group. This enrollment group will have a set of credentials for all IoT devices in the same group.

Select Manage enrollments tab and click + Add enrollment group.

- On Add Enrollment Group, enter anomaly-detection-group as a Group name

- Select Symmetric Key as Attestation Type

- Check Auto-generate keys checkbox and click Save.

Once you saved your enrollment group, open the enrollment, copy, and save the value of your generated Primary Key for future usage. This key is your master group key – we will need it in our IoT device simulator.

Another important parameter, needed for our device simulator is DPS ID Scope. Go to Overview tab and find ID Scope value:

Copy and save it somewhere in your notes as DPS ID Scope value.

## 4. Create an IoT device simulator

In this section we will create an IoT device simulator sending telemetry data (temperature and humidity) to the IoT hub.

First, you can use Git to clone the repository with the source code. Change the current working directory to the location where you want the cloned directory. Then, type:

> git clone https://github.com/jevgenij-p/iot-workshop-anomaly-detection.git

Downloaded repository contains source folder with two sub-folders:

source\simulator\python
source\simulator\csharp

You can choose the version of the IoT device simulator: either written in Python, or C#. We will discuss both.

### 4.1 IoT device simulator. Python version

This is a small application, connecting to an IoT hub, registering a device, and sending some telemetry to the hub. The telemetry contains temperature and humidity values, changing in a narrow range. We can increase or decrease temperature values pressing “+” or “-“ keys and ENTER key, or to quit the application pressing the “q” key.

But before running the simulator ensure that your system meets all prerequisites.

#### 4.1.1 Prerequisites

Install the latest version of Python and add it to the Windows Path variable.

Change the current working directory to the source\simulator\python directory, and create a virtual environment:

> python -m venv env

Activate the virtual environment by the following command:

> .\env\Scripts\activate

Install required packages:

(env) > pip3 install azure-iot-device
(env) > pip3 install asyncio

#### 4.1.2 Run IoT device simulator

Before you run the application, you shall set some system variables. Please find the Primary Key and DPS ID Scope you saved before. You need to replace [Primary_Key] and [DPS_ID_Scope] with the saved values in the following commands:

(env) > SET DPS_DEVICE_ID=device001

(env) > SET DPS_ID_SCOPE=[DPS_ID_SCOPE]

(env) > SET DPS_PRIMARY_KEY=[PRIMARY_KEY]

For example:

(env) > SET DPS_DEVICE_ID=device001
(env) > SET DPS_ID_SCOPE=0ne004269B3

(env) > SET DPS_PRIMARY_KEY=cwB2AGMAXgBbAC8ASQA8AG8AJQBsAHwAeQBoACQAKQA3ADAAOABlADsAZQBpACsAQwBpAFAAdABfAEkAVQByAA==

Run the simulator:

(env) > python telemetry.py

If everything is set up correctly, you will see messages sent to the IoT hub every two seconds:

If you are not familiar with Python, you can use C# version of the simulator.

### 4.2 IoT device simulator. C# version

The source code of the C# simulator is in the source\simulator\csharp\telemetry directory. The logic of this application is absolutely the same as of its Python version: it connects to an IoT hub, registers a device, and sends telemetry to the hub. We can increase or decrease temperature values pressing “+” or “-“ keys and ENTER key, or to quit the application pressing Ctrl+C.

But before running the simulator check out the following prerequisites.

#### 4.2.1 Prerequisites

Install Visual Studio Code first and then add the following extensions:

- C# for Visual Studio Code

- Azure IoT Tools for Visual Studio Code (optional)

The IoT device simulator is a .NET 5.0 console application. So, the .NET Runtime 5.0 shall be installed on your workstation.

Open a Command Prompt window. Change the current working directory to the source\simulator\csharp\telemetry directory, and restore the dependencies of the project:

> dotnet restore

#### 4.2.2 Run IoT device simulator

Before you run the application, you shall set some system variables. Please find the Primary Key and DPS ID Scope you saved before. You need to replace [Primary_Key] and [DPS_ID_Scope] with the saved values in the following commands:

> SET DPS_DEVICE_ID=device001

> SET DPS_ID_SCOPE=[DPS_ID_SCOPE]

> SET DPS_PRIMARY_KEY=[PRIMARY_KEY]

For example:

> SET DPS_DEVICE_ID=device001
> SET DPS_ID_SCOPE=0ne004269B3

> SET DPS_PRIMARY_KEY=cwB2AGMAXgBbAC8ASQA8AG8AJQBsAHwAeQBoACQAKQA3ADAAOABlADsAZQBpACsAQwBpAFAAdABfAEkAVQByAA==

Run the simulator:

> dotnet run

The simulator will be sending messages to the IoT hub every two seconds:

Press “+” or “-“ and ENTER to increase or decrease temperature. Press Ctrl+C to quit the application.

## 5. Use Azure IoT Explorer to view the telemetry

It looks our simulator is sending some data to the IoT hub. But it would be great to see what exactly it is sending. Fortunately, there’s a tool to view the telemetry the devices are sending: Azure IoT Explorer. Download and install its latest version.

To establish connection with your IoT hub, you will need to get an IoT hub connection string:

- In the Azure portal, select anomaly-detection-hub

- Select Shared access policies tab

- Click on iothubowner policy name to open its properties

- Copy Primary connection string and save it in your notes

Open the Azure IoT Explorer, select IoT hubs tab, add the connection string you have just copied, and click Save.

After the Azure IoT Explorer connects to your IoT hub, it displays the devices registered with your IoT hub. Select device001 in the list and click Telemetry tab on the left:

Ensure your IoT device simulator is running and click Start to browse received events. If Show system properties checkbox is unchecked, you will see received messages like this:

{

"body": {

"temperature": 20.08,

"humidity": 59.59

},

"enqueuedTime": "Wed Dec 01 2021 13:38:51 GMT+0200 (Eastern European Standard Time)",

"properties": {}

}

Check Show system properties checkbox to see all message properties:

{

"body": {

"temperature": 19.23,

"humidity": 60.59

},

"enqueuedTime": "Wed Dec 01 2021 13:38:35 GMT+0200 (Eastern European Standard Time)",

"properties": {},

"systemProperties": {

"iothub-connection-device-id": "device001",

"iothub-connection-auth-method": "{\"scope\":\"device\",\"type\":\"sas\",\"issuer\":\"iothub\",\"acceptingIpFilterRule\":null}",

"iothub-connection-auth-generation-id": "637739461643697540",

"iothub-enqueuedtime": 1638358715950,

"iothub-message-source": "Telemetry",

"contentType": "application/json",

"contentEncoding": "utf-8"

}

}

Now you know how to create an IoT hub, connect IoT devices to it using Device Provisioning service, send telemetry, and browse sent messages in the Azure IoT Explorer. Further we will talk about how to visualize telemetry data.

# Hands-on Lab. Part II

## 6. Create a Stream Analytics job

We are going to receive telemetry stream (temperature and humidity) and detect in real time sudden temperature spikes or dips, using Stream Analytics Query Language.

Azure Stream Analytics offers built-in machine learning based AnomalyDetection_SpikeAndDip function (see Anomaly detection in Azure Stream Analytics) which returns two columns:

IsAnomaly - a BIGINT (0 or 1) indicating if the event was anomalous or not, and
Score - the computed p-value score (float) indicating how anomalous an event is.

We will output the results into an Event Hub so the events corresponding to anomalies can be consumed by other components: Time Series Insights, Power BI, and Azure Functions.

Now, we will set up a Stream Analytics job with this anomaly detection function to read telemetry from our IoT Hub and detect anomalies.

- In the Azure portal, select + Create a resource, and select Stream Analytics job

- Enter streamjob as a Job name

- Select your Subscription, select anomaly-detection-rg as a Resource group, and North Europe as a Region

- Click Create.

Once our Stream Analytics job is created, we need to configure it – create an Input, Output, and a Query.

### 6.1 Configure Stream Analytics Input

Our next step is to define an input source for the job to read data from the IoT hub.

- Select Inputs tab

- Click + Add stream input and select IoT Hub. IoT Hub panel will be opened (see figure below)

- Enter input as Input alias

- Select your Subscription

- Select anomaly-detection-hub as an IoT Hub

- Select asaconsumergroup as a Consumer group

- Select iothubowner as a Shared access policy name

- Click Save

We want to detect anomalies in temperature and feed these anomalous “events” into other systems or services: Time Series Insights and Azure Functions. Stream Analytics supports many types of outputs: Azure Data Lake, Azure SQL Database, Azure Synapse Analytics, Blob and Table storages, Event Hub, Service Bus, Power BI, Cosmos DB, and Azure Functions. We will use two Event Hubs as outputs for our job. One – for Time Series Insights, and second – for an Azure Function.

### 6.2 Configure Stream Analytics Outputs

First, we need to create a new Event Hub namespace in which we create our event hubs.

- Go to anomaly-detection-rg resource group, click + Create, and select Event Hubs

- Select your Subscription, select anomaly-detection-rg as a Resource group, and select North Europe as Location

- Enter a unique name for the Namespace name, and then select Review + Create

- On the Review + Create page, select Create.

Once the deployment is complete, select Go to resource to open the Event Hubs Namespace. We will create two event hubs: eventhub1 as a data source for Time Series Insights, and eventhub2 as a data source for Azure Functions.

- Select Event Hubs tab and click + Event Hub

- Enter eventhub1 as a Name and click Create.

- Select Event Hubs tab and click + Event Hub

- Enter eventhub2 as a Name and click Create.

To provide access to the Time Series Insights, we will need to create a Shared Access Policy.

- Select Shared access policies tab and click + Add

- Enter eventHubSharedAccessPolicy as a Policy name

- Check Manage checkbox and click Create.

Now we have two event hubs, which will be used as Stream Analytics job outputs.

- Select Outputs tab, click + Add, and select Event Hub

- Enter output1 as an Output alias

- Select your Subscription

- Select the name of the Event Hub namespace your created

- Select eventhub1 as an Event Hub name

- Select Connection string as an Authentication mode

- Select eventHubSharedAccessPolicy as an Event Hub policy name and click Save.

Repeat the same steps for the second output:

- Click + Add, and select Event Hub

- Enter output2 as an Output alias

- Select your Subscription

- Select the name of the Event Hub namespace your created

- Select eventhub2 as an Event Hub name

- Select Connection string as an Authentication mode

Select eventHubSharedAccessPolicy as an Event Hub policy name and click Save.

### 6.3 Create Stream Analytics query

We will be using a Stream Analytics job to send telemetry data to two different outputs using a SQL-like query language (check out Stream Analytics Query Language Reference to learn more about that).

If we would need to send data to multiple output targets without any transformations, we could use a query like:

SELECT

*

INTO output1

FROM input;

SELECT

*

INTO output2

FROM input;

Where input is the Input alias, we created on the step Configure Stream Analytics Input, which is our IoT hub, in fact. output1 and output2 are Output aliases, pointing to eventhub1 and eventhub2. If we run this query, telemetry stream will be sent to these event hubs.

IoT hub telemetry stream contains many properties (see Configure an IoT Hub as a data stream input). If we need to filter only some of them, we could create a query like this:

SELECT

IoTHub.ConnectionDeviceId AS id,

EventEnqueuedUtcTime AS time,

CAST(temperature AS float) AS temperature

INTO output1

FROM input;

But our goal is to detect anomalies in temperature. The solution for this use-case is AnomalyDetection_SpikeAndDip function. We can write a query:

SELECT

IoTHub.ConnectionDeviceId AS id,

EventEnqueuedUtcTime AS time,

CAST(temperature AS float) AS temperature,

AnomalyDetection_SpikeAndDip(CAST(temperature AS float), 95, 120, 'spikesanddips')

OVER(PARTITION BY id LIMIT DURATION(second, 240)) AS SpikeAndDipScores

INTO output1

FROM input;

Where we pass four parameters to the AnomalyDetection_SpikeAndDip function:

| CAST(temperature AS float) | the event column, containing temperature |
| --- | --- |
| 95 | confidence - a percentage number from 1.00 to 100. The lower the confidence, the higher the number of anomalies detected, and vice versa |
| 120 | historySize - the number of events in a sliding window that the model continuously learns from and uses for scoring the next event for anomalousness |
| 'spikesanddips' | mode - a string parameter whose value is "spikes", "dips", or "spikesanddips" |

Maybe we will need to modify these parameters later, but let’s leave it so far.

The following expression

OVER(PARTITION BY id LIMIT DURATION(second, 240)) AS SpikeAndDipScores

specifies the size of the sliding window within Stream Analytics in terms of time. The recommended size of this time window is the equivalent of the time it takes to generate historySize number of events in steady state (120 events * 2 sec/per event = 240 seconds).

The AnomalyDetection_SpikeAndDip function returns a record containing the following columns:

| IsAnomaly | A BIGINT (0 or 1) indicating if the event was anomalous or not |
| --- | --- |
| Score | The computed p-value score indicating how anomalous an event is. Lower scores mean a higher probability that the event is anomalous |

We will use the following query:

WITH AnomalyDetectionStep AS

(

SELECT

IoTHub.ConnectionDeviceId AS id,

EventEnqueuedUtcTime AS time,

CAST(temperature AS float) AS temperature,

CAST(humidity AS float) AS humidity,

AnomalyDetection_SpikeAndDip(CAST(temperature AS float), 95, 120, 'spikesanddips')

OVER(PARTITION BY id LIMIT DURATION(second, 240)) AS SpikeAndDipScores

FROM input

WHERE temperature IS NOT NULL

)

SELECT

id AS 'device-id',

time,

temperature,

humidity,

CAST(GetRecordPropertyValue(SpikeAndDipScores, 'IsAnomaly') AS bigint) AS temperature_anomaly,

CAST(GetRecordPropertyValue(SpikeAndDipScores, 'Score') AS float) AS temperature_anomaly_score

INTO output1

FROM AnomalyDetectionStep;

SELECT

id AS 'device-id',

time,

temperature,

CAST(GetRecordPropertyValue(SpikeAndDipScores, 'IsAnomaly') AS bigint) AS temperature_anomaly,

CAST(GetRecordPropertyValue(SpikeAndDipScores, 'Score') AS float) AS temperature_anomaly_score,

ISFIRST(mi, 1) OVER(PARTITION BY id WHEN CAST(GetRecordPropertyValue(SpikeAndDipScores, 'IsAnomaly') AS bigint) = 1) AS first

INTO output2

FROM AnomalyDetectionStep

WHERE CAST(GetRecordPropertyValue(SpikeAndDipScores, 'IsAnomaly') AS bigint) = 1

Figure 2. Stream Analytics job query

which will send events containing the following columns to event hubs:

| device-id | Device ID |
| --- | --- |
| time | The date and time that the event was received by the IoT Hub |
| temperature | Temperature |
| humidity | Humidity |
| temperature_anomaly | An integer, 0 or 1, indicating whether the event is anomalous or not |
| temperature_anomaly_score | A score indicating how anomalous the event is |
| first | An integer. 1 if the event is the first event within a given fixed interval, or 0 otherwise |

Now we are ready to configure a query in the Stream Analytics job.

- Select Query tab of the job

- Paste the query we prepared (Figure 2) to the query window and click Save query

To ensure that everything is configured correctly, we can test our query. Start your IoT device simulator and click Test query. If you do not see test results – do not worry – it can take a few minutes to receive events from input.

We just configured the Stream Analytics job. The job is not running yet – we need to start it. Select Overview tab and click Start. The Stream Analytics job will start processing telemetry data and writing query results to the Event Hubs we created early.

## 7. Create Time Series Insights

We can use Azure Time Series Insights (TSI) to visualize telemetry data that is generated by IoT devices. It can be easily connected to IoT Hubs or Event Hubs to present how their messages change over time. TSI provides visualization through the Azure Time Series Insights Explorer. But first, we need to create a TSI service.

- In the Azure portal, select + Create a resource, and select Time Service Insights

- Select your Subscription, select anomaly-detection-rg as a Resource group, and North Europe as a Region

- Enter time-series-insights as an Environment name

- Enter device-id as a Property name (we specified it in our Stream Analytics job query)

- Enter a unique Storage account name, and then select Next: Event Source

- Enter ioteventsource as a Name

- Select the Event Hub namespace you created as a container for Event hubs in the Configure Stream Analytics outputs section

- Select eventhub1 as an Event Hub name

- Select eventHubSharedAccessPolicy as an Event Hub access policy name

- Select $Default as an Event Hub consumer group, and then select Review + Create

- On the Summary page, select Create.

Once the TSI is created, open it.

## 8. Visualize telemetry data in Time Series Insights

The TSI Explorer is a graphical tool visualizing telemetry using different chart types. We can define a hierarchy for the telemetry data ingested. We can contextualize the telemetry applying a Time Series Model to simplify finding and analyzing Time Series data.

Check that your IoT device simulator is still running, select Overview tab in Time Series Insights and click Go to TSI Explorer.

The TSI Explorer shall display only one Time Series instance for device001 – our simulated IoT device ID:

- Select device001 – a pop up menu with available streams will be displayed

- Add humidity, temperature, temperature_anomaly and temperature_anomaly_score.

All four streams shall be displayed. But since the temperature is varying in a narrow range, the temperature_anomaly will be zero.

We can change this situation. Switch to the console application, running your IoT device simulator and try to increase or decrease temperature with “+” and “-“ keys:

You should see (with some time lag) how temperature_anomaly and temperature_anomaly_score is changing over time.

The temperature_anomaly = 1 indicates the temperature is anomalous. Zero level means normal temperature.

## 9. Visualize data from Time Series Insights in Power BI

Azure Time Series Insights is an analytics, storage, and visualization service that allows to explore billions of IoT events simultaneously. Power BI is more familiar tool for business needs, for users working with IoT data to create reports and dashboards.

The TSI Explorer provides a Power BI Connector, which supports export both your recent and historical data out to Power BI. Let me illustrate it on a small example.

- Open the TSI Explorer, displaying your telemetry data

- Navigate to the More actions dropdown menu and select Connect to Power BI

The TSI Explorer generates a Custom query for the Power BI Connector to use. See more about how to use the Power BI Connector in the article Visualize data from Azure Time Series Insights in Power BI.

- Choose a Store type: Warm Store for the latest data or Cold Store for historical data.

- Click Copy query to clipboard

- Run your Power BI Desktop application and select Home tab

- Select Get Data, then select More

- Search for Time Series, select Azure Time Series Insights (Beta), and click Connect

- Paste the query you copied from the TSI Explorer into the Custom Query field, then click OK

- Press Load to load data into Power BI

When you have imported the data into Power BI, you can create a report, for example, like this one:

Once you have created your report, you can save it or publish it to Power BI Reporting Services to share it with others.

# Hands-on Lab. Part III

## 10. Create a script, deploying Azure resources

As you already noticed, manual creating of Azure resources is boring and takes too much time. To automate this process, we can use Azure CLI commands for Azure IoT. We can create and configure our IoT Hub, Stream Analytics and Time Series Insights using a single PowerShell script, deploying all the resources to the cloud. But before we run it, we need to check some prerequisites.

### 10.1 Prerequisites

We will need the following software:

- The latest version of PowerShell on Windows

- Azure CLI on Windows

- Microsoft Azure IoT extension for Azure CLI

- PowerShell Language Support for Visual Studio Code

I recommend using Visual Studio Code for work with PowerShell scripts.

### 10.2 Deployment script

Change the current working directory to the source\scripts folder. There are two files:

- deployment.ps1

- functions.ps1

Open the files in an editor. The functions.ps1 is an auxiliary script with PowerShell functions, generating random strings. deployment.ps1 is more interesting for us.

This is a script deploying the following resources to the cloud:

- Resource group anomaly-detection-rg

- IoT Hub anomaly-detection-hub

- IoT Hub device provisioning service anomaly-detection-dps

- Stream Analytics job streamjob

- Time Series Insights Gen2 time-series-insights

Before you run the script, open Azure portal, and make sure that the anomaly-detection-rg resource group does not exist. If it exists – delete it.

Open a Command Prompt window and run the command:

> pwsh .\deployment.ps1

The Azure CLI will open your default browser and load an Azure sign-in page. Sign in with your Azure account credentials in the browser.

After successful sign-in, the script will create required resources in the anomaly-detection-rg resource group. That can take a few minutes (around eight minutes in my case). You can take a cup of coffee while robots are working .

In the end, the script will print out the DPS ID Scope and DPS Primary Key, which is used in our IoT device simulator (see section Run IoT device simulator).

To check that everything is created, open the anomaly-detection-rg resource group in Azure portal and select Overview tab. You shall see resources, created by the script.

Now, you can run your IoT device simulator, experiment with the Stream Analytics, Time Series Insights or Power BI. When you finish your experiments, I recommend you delete the anomaly-detection-rg resource group to clean up the Azure resources to avoid extra costs.

# Hands-on Lab. Part IV

## 11. Create an Azure Function

Let us create an Azure Function that would send out email notification when the telemetry value (temperature) sent out by our simulated IoT device is anomalous.

### 11.1 Prerequisites

We will create this function using Visual Studio Code. You should install Node.JS which includes npm.

Then, run the following command to install the Azure Functions Core Tools package:

> npm install -g azure-functions-core-tools@3 --unsafe-perm true

We will need an Azure Functions extension for Visual Studio Code to develop and deploy functions to Azure.

We already have such a function in the source\functionapp directory. But we will use this code just as an example and create a new Azure Function in another directory from scratch.

Before we continue, ensure all required Azure resources (IoT hub, DPS, TSI, and so on) exist in the anomaly-detection-rg resource group in the Azure portal. If not, recreate them manually, or using the PowerShell script (deployment.ps1).

### 11.2 Create an Azure Function App in the portal

First, we must have a function app to host the execution of our functions. A function app lets us run our code in a serverless environment.

- Open Azure portal on the browser

- Go to anomaly-detection-rg resource group, click + Create, and select Function App

- Select your Subscription, select anomaly-detection-rg as a Resource group

- Enter a unique name (like anomalyeventtrigger in this example) as a Function App name

- Select .NET as Runtime stack and 3.1 as Version

- Select North Europe as Region

- Click Review + Create

- On the Review + Create page, select Create.

Once the deployment is complete, select Go to resource to open new Function App.

### 11.3 Create an Event Hub Trigger

Now we will learn how to create an Azure Event Hubs trigger. The function trigger will respond to an event sent to the eventhub2 stream. When the function is triggered, the message passed to the function is typed as a string.

Open Visual Studio Code and select Azure tab on the left navigation menu.

Select your Azure subscription. You shall see a function app you just created.

- Click the Create New Project… icon  in the Functions panel and select an empty folder (for example, myapp) that will contain your function  project

- Select a language C#

- Select a .NET Runtime .NET Core 3 LTS

- Select a template for your project’s first function Azure Event Hub trigger

- Provide a function name SendEmailOnEventTrigger

- Provide a namespace IoTLab.Workshop

- Select + Create new local app setting

- Select an event hub namespace

- Select an event hub eventhub2

- Select an event hub policy eventHubSharedAccessPolicy

- Skip the step, selecting a storage account for internal use.

New local project will be created in the selected folder. If you do not see it in the Azure: Functions panel, click Refresh icon.

- Open created SendEmailOnEventTrigger.cs in the Visual Studio Code

- Find and open source/functionapp/SendEmailOnEventTrigger.cs in an editor. Copy its content to the created myapp/SendEmailOnEventTrigger.cs

Open the myapp/SendEmailOnEventTrigger.cs file and check it.

When a batch of new messages is received from an event hub, the host attempts to trigger the function calling the Run() method. Each event hub message will be sent to an email account. If you look at the SendEmail() method, you will see that we are sending an email to a Gmail account:

using (var client = new SmtpClient())

{

client.UseDefaultCredentials = false;

client.Credentials = new System.Net.NetworkCredential(username, password);

client.Port = 587;

client.Host = "smtp.gmail.com";

client.DeliveryMethod = SmtpDeliveryMethod.Network;

client.EnableSsl = true;

await client.SendMailAsync(msg);

}

Personally, I have a Gmail account and want to use it to test this function app. If you prefer to use another mail server – modify this code accordingly.

The last thing to do is to build and publish the function.

- Open a Command Prompt window (or a Terminal window in the Visual Studio Code)

- Change the current working directory to the myapp (or other directory, containing your new function app)

- Run the code to build the project:

> dotnet restore

> dotnet build

If there are no errors, click Deploy to Function App… icon to publish it to the cloud:

# References

- Download Python

- Visual Studio Code

- Overview of Azure IoT Device SDKs

- Azure IoT Samples for C# (.NET)

- Azure IoT Explorer (preview)

- Download Azure IoT Explorer

- Quickstart: Set up the IoT Hub Device Provisioning Service with the Azure portal

- What is Azure IoT Hub Device Provisioning Service?

- How to provision devices using symmetric key enrollment groups

- Quickstart: Create a Stream Analytics job by using the Azure portal

- Anomaly detection in Azure Stream Analytics

- AnomalyDetection_SpikeAndDip Azure Stream Analytics function

- Stream Analytics Query Language Reference

- Create an Azure Time Series Insights Gen2 environment using the Azure portal

- Azure Time Series Insights Explorer

- Visualize data from Azure Time Series Insights in Power BI

- Azure IoT CLI

- Azure CLI reference commands for Azure IoT

- Install Azure CLI on Windows

- Microsoft Azure IoT extension for Azure CLI

- Installing PowerShell on Windows

- PowerShell Language Support for Visual Studio Code

- Sign in with Azure CLI

- Develop Azure Functions by using Visual Studio Code

- Downloading and installing Node.js and npm

- Azure Functions for Visual Studio Code

- Azure Event Hubs trigger for Azure Functions